Performance Task

FM Sidebands

Name: Jhon Paulo V. Gascon        Course/Section: BSIT

Given: Modulating frequency (fm) = 4 kHz          Carrier frequency (fc) = 20 kHz          Maximum frequency deviation (Δf) = 4 kHz

a. Value of Four (4) Sideband Pairs

Sideband frequencies are found using: fc ± (n × fm), for n = 1 to 4

| Pair | Lower Sideband (fc − n·fm) | Upper Sideband (fc + n·fm) | Amplitude |
| --- | --- | --- | --- |
| n = 1 | 16 kHz | 24 kHz | J1 = 0.44 |
| n = 2 | 12 kHz | 28 kHz | J2 = 0.11 |
| n = 3 | 8 kHz | 32 kHz | J3 = 0.02 |
| n = 4 | 4 kHz | 36 kHz | J4 = 0.00 |

b. Frequency Deviation

The maximum frequency deviation is given directly in the problem:

Answer: Δf = 4 kHz

c. Modulation Index

mf = Δf / fm

mf = 4 kHz / 4 kHz = 1

Answer: mf = 1

d. FM Spectrum (Bessel Function Table, mf = 1)

Using the Bessel function table at mf = 1: J0 = 0.77, J1 = 0.44, J2 = 0.11, J3 = 0.02, J4 = 0.00